IOP Test Plan

EMSCI Driver

| Project Name | IOP Extension Package |
| --- | --- |
| Project ID |  |
| Document ID | IOP_EMSCI_TEST_CASES.doc |
| Version | 1.0 |
| Date | September 25th, 2008 |
| Status | Waiting for approval |
| iCAP Classification | Freescale Internal Use Only |

| Approval list |  |  |
| --- | --- | --- |
| Role | Name | Date |

Version Tracking

| Date | Version | Comment | Author |
| --- | --- | --- | --- |
| September 25, 2008 | 0.1 | Initial draft | R01160 |

Table of Contents

1.	Scope of this document	1

2.	Acronyms and definitions	1

3.	General test suite conditions	1

3.1.	Environment prerequisites (installation and network)	1

3.2.	Test pre-conditions (data created before testing)	1

3.3.	Other sources and prerequisites information	2

3.4.	Test Cases enablement	2

3.5.	Tables & Diagrams	2

# Scope of this document

This document describes in detail the test plan description of Emulated SCI driver for 5510 family using the Zen core (Z0) to meet customer requirements. For each test of a particular requirement, an expected state/response is detailed. The result of expected state/response is the acceptance criteria.

Reference Documents:

MPC5510_Reference_Manual_Rev0.pdf

| Type of Test Case | Test Case ID |
| --- | --- |
| Customer | EMSCI_TCxxx |

Table 1.0. Test Case Types

# Acronyms and definitions

| Acronyms | Definitions |
| --- | --- |
| IOP | Input/Output Processor |
| Z0 | Identical Second core of the MPC5510 |
| EMSCI | Emulated SCI |

# General test suite conditions

The EMSCI drivers will be tested for Freescale’s 5510 microcontroller family according to Customer PWM driver specifications. A CodeWarrior v2.3 will be used for testing.

IMPORTANT

It is required to have an SCI transceiver to perform all test cases proposed within this document.

## Environment prerequisites (installation and network)

- MPC5516 Evaluation board.

- Lauterbach Debugger with Trace32 Software preinstalled or P&E Multilink.

- CodeWarrior v2.3.

- 4 Oscilloscope probes, ONE 4-Ch Digital oscilloscope.

## Test pre-conditions (data created before testing)

- Create a complete test plan for the emulated SCI module.

- Have a complete set of test cases which will let final tester to run every test specified

in test plan.

- Test cases source code.

- Test case document to report final results of each test.

## Other sources and prerequisites information

NA

## Test Cases enablement

Each Test case MUST be enabled when no other test case is activated. The enablement of two test cases at a time WILL cause some errors or malfunctions in the operation of the EMSCI driver.

To activate a test case in the source code, user shall go directly to code file named “Emsci_Cfg.h” and switch the current state from the desired Test case to be activated from “OFF” to “ON”.

## Tables & Diagrams

4. Test Cases

| TC ID | EMSCI_TC001 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 20 min |
| Objectives |  | Test EMSCI Error layer throughout the use of wrong configuration parameters and null pointer structures |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Det error layer shall be available within the context of this driver. |  |  | 1) vfnEmsci_Report_Error function was called with parameters EMSCI_INIT_ID(0x00), EMSCI_E_PARAM_CONFIG (0x10). | 1) Enable Development Error Detection switch before compilation. Located at Emsci_Cfg.h file. “EMSCI_ERROR_DETECT ON” 2) Call Emsci_Init function with Null pointer as the parameter. |  | 1) “vfnEmsci_Report_Error” function should be called with parameters EMSCI_INIT_ID(0x00), EMSCI_E_PARAM_CONFIG (0x10). Check variables “Emsci_ApiId” and “Emsci_ErrorId” 2) Initialization of the Emsci structure should be skipped. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI _TC002 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 10min |
| Objectives |  | Turn EMSCI Error layer off to not validate wrong parameters when vfnEmsci_Init be invoked |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Det error layer shall be available within the context of this driver. |  |  | Variable Type: Emsci_ConfigType* ConfigPtr | - Disable Development Error Detection switch before compilation. - Call Emsci_Init Function, Pass pointer (properly initialized) as input parameter. |  | 1 ) vfnEmsci_Report_Error Function should not be called. 2) All the EMSCI related Parameters from Emsci_Init function shall initialize the drver according to the EMSCI config container values. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI _TC003 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 20 min |
| Objectives |  | Test the correct passing of parameters from “vfnEmsci_Init” function into “vfnEmios_Init” function. |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) GPIO pins available for configuration. |  |  | 1) EMSCI configuration structure properly initialized. | - Enable Development Error Detection switch before compilation. - Call vfnEmsci_Init Function, Pass pointer (properly initialized) as input parameter. - Debug until the end of “vfnPort_Init” function be Reached. |  | 1)vfnEmsci_Report_Error Function should not be invoked. 2) All the EMSCI related configuration parameters should be correctly passed to the “vfnEmsci_Init” initialization function. 3) Pins 0 & 1 from port B shall be configured as outputs and pins shall be enabled emios pin functionality. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI _TC004 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 10 min |
| Objectives |  | Validate the configuration of the only configured resources stated in the “Emsci_Cfg.c” file. |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Preconfigure parameter configuration file shall exist. 2) At least one SCI channels shall be properly configured. |  |  | 1) EMSCI configuration pointer properly initialized (EmsciCfgPtr). - One configured SCI channels with the follow parameters: 2.1) SCI Channel 0: 2.1.1) Emios 0 as TX 2.1.2) Emios 0 Interrupt 58. 2.1.3) Port pin 32 2.1.4) Emios 1 as RX 2.1.2) Emios 1 Interrupt 59. 2.1.3) Port pin 33 | - Enable “EMSCI_ERROR_DETECT” switch before compilation. - Call Emsci_Init Function using Emsci configuration pointer. 3) Run through Emsci_Init fucntion. - Validate every configuration parameter from EmsciCfgPtr to be written correctly into Emios channels 0 & 1 correctly. This procedure must be done by visual inspection. |  | 1)vfnEmsci_Report_Error Function should not be called. 2) Emios channels 0 & 1 must be correctly configured and initialized. 3) No other Emios channels rather than channels 0 & 1 shall be configured. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC005 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 20 min |
| Objectives |  | Test and Validate the correct frame generation at 2400 bps |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 0 available. |  |  | 1) EMSCI_BUS_FREQUENCY definition set at 2400. | - Enable “EMSCI_ERROR_DETECT” switch before compilation (Emsci_Cfg.h). - Call Emsci_Init Function using Emsci configuration pointer. - Configure a GPIO pin as an Output (PC0). 4) Enable pin functionality to operate as Emios mode with Output compare mode enabled. - Run the EMSCI driver. - Set an oscilloscope probe at pin 32 from port C. (PC0) |  | 1)vfnEmsci_Report_Error Function should not be called. 2) Bit rate period at 416us. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC006 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 20 min |
| Objectives |  | Test and Validate the correct frame generation at 9600 bps |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 0 available. |  |  | 1) EMSCI_BUS_FREQUENCY definition set at 9600. | - Enable “EMSCI_ERROR_DETECT” switch before compilation (Emsci_Cfg.h). - Call Emsci_Init Function using Emsci configuration pointer. - Configure a GPIO pin as an Output (PC0). 4) Enable pin functionality to operate as Emios mode with Output compare mode enabled. - Run the EMSCI driver. - Set an oscilloscope probe at pin 32 from port C. (PC0) |  | 1)vfnEmsci_Report_Error Function should not be called. 2) Bit rate period at 104us. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC007 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 20 min |
| Objectives |  | Test and Validate the correct frame generation at 19200 bps |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 0 available. |  |  | 1) EMSCI_BUS_FREQUENCY definition set at 19200. | - Enable “EMSCI_ERROR_DETECT” switch before compilation (Emsci_Cfg.h). - Call Emsci_Init Function using Emsci configuration pointer. - Configure a GPIO pin as an Output (PC0). 4) Enable pin functionality to operate as Emios mode with Output compare mode enabled. - Run the EMSCI driver. - Set an oscilloscope probe at pin 32 from port C. (PC0) |  | 1)vfnEmsci_Report_Error Function should not be called. 2) Bit rate period at 52us. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC008 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 20 min |
| Objectives |  | Test and Validate the correct frame generation at 19200 bps |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 0 available. |  |  | 1) EMSCI_BUS_FREQUENCY definition set at 19200. | - Enable “EMSCI_ERROR_DETECT” switch before compilation (Emsci_Cfg.h). - Call Emsci_Init Function using Emsci configuration pointer. - Configure a GPIO pin as an Output (PC0). 4) Enable pin functionality to operate as Emios mode with Output compare mode enabled. - Run the EMSCI driver. - Set an oscilloscope probe at pin 32 from port C. (PC0) |  | 1)vfnEmsci_Report_Error Function should not be called. 2) Bit rate period at 52us. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC009 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 20 min |
| Objectives |  | Configure and Validate by visual inspection the correct generation of a serial frame using TX unit 0. |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 0 available. 2) Pin 32 in PortC available. |  |  | 1) EMSCI configuration structure properly initialized. 2) Pin 32 in PortC configured to operate in Output Compare Mode. | - Pin 32 in PortC configured to operate in Output Compare Mode. - Invoke the macro named “EMSCI_TX” using the below parameters: 2.1) Serial Unit number 0 2.2) 0x55. 3) Run EMSCI driver. - Set an oscilloscope probe at PortC pin 32. |  | 1)vfnEmsci_Report_Error Function should not be called. 2) PortC pin 32 shall generate a complete serial frame. Start and Stop bits shall be generated as well. A 0x55 data shall be generated. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC010 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 20 min |
| Objectives |  | Configure and Validate by visual inspection the correct generation of a serial frame using TX unit 1. |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 2 available. 2) Pin 34 in PortC available. |  |  | 1) EMSCI configuration structure properly initialized. 2) Pin 34 in PortC configured to operate in Output Compare Mode. | - Pin 34 in PortC configured to operate in Output Compare Mode. - Invoke the macro named “EMSCI_TX” using the below parameters: 2.1) Serial Unit number 1 2.2) 0x15. 3) Run EMSCI driver. - Set an oscilloscope probe at PortC pin 34. |  | 1)vfnEmsci_Report_Error Function should not be called. 2) PortC pin 34 shall generate a complete serial frame. Start and Stop bits shall be generated as well. A 0x15 data shall be generated. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC011 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 20 min |
| Objectives |  | Configure and Validate by visual inspection the correct generation of a serial frame using TX unit 2. |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 4 available. 2) Pin 36 in PortC available. |  |  | 1) EMSCI configuration structure properly initialized. 2) Pin 36 in PortC configured to operate in Output Compare Mode. | - Pin 36 in PortC configured to operate in Output Compare Mode. - Invoke the macro named “EMSCI_TX” using the below parameters: 2.1) Serial Unit number 2 2.2) 0x25. 3) Run EMSCI driver. - Set an oscilloscope probe at PortC pin 36. |  | 1)vfnEmsci_Report_Error Function should not be called. 2) PortC pin 36 shall generate a complete serial frame. Start and Stop bits shall be generated as well. A 0x25 data shall be generated. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC012 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 20 min |
| Objectives |  | Configure and Validate by visual inspection the correct generation of a serial frame using TX unit 3. |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 6 available. 2) Pin 38 in PortC available. |  |  | 1) EMSCI configuration structure properly initialized. 2) Pin 38 in PortC configured to operate in Output Compare Mode. | - Pin 38 in PortC configured to operate in Output Compare Mode. - Invoke the macro named “EMSCI_TX” using the below parameters: 2.1) Serial Unit number 3 2.2) 0x35. 3) Run EMSCI driver. - Set an oscilloscope probe at PortC pin 38. |  | 1)vfnEmsci_Report_Error Function should not be called. 2) PortC pin 38 shall generate a complete serial frame. Start and Stop bits shall be generated as well. A 0x35 data shall be generated. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC013 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 30 min |
| Objectives |  | Configure and Validate the RX functionality of RX Unit 0. Validate the correct reception of all bits on Emios Channel 1. |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 0 & 1 available. 2) Pin 32 & 33 in PortC available. 3) SCI Transceiver needed to corroborate the operation of reception bits. 4) Configure a Loop Back mode between TX and RX channels (Pins 32 & 33). |  |  | 1) EMSCI configuration structure properly initialized. 2) Pin 32 in PortC configured to operate in Output Compare Mode. 3) Pin 33 in PortC configured to operate on Input Capture Mode. | - Pin 32 in PortC configured to operate in Output Compare Mode. - Invoke the macro named “EMSCI_TX” using the below parameters: 2.1) Serial Unit number 2 2.2) 0x10. 3) Configure PortC pin 33 to operate in “Input Capture” mode. - After 100ms invoke macro function named “EMSCI_RX” using the below parameters: 4.1) SCI channel 0 4.2) Create a temporal variable named “u8RxData” to save any incoming data. 5) Run EMSCI driver. |  | 1)vfnEmsci_Report_Error Function should not be called. 2) After 100ms the variable named “u8RxData” shall have the value of 0x10. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC014 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 30 min |
| Objectives |  | Configure and Validate the RX functionality of RX Unit 1. Validate the correct reception of all bits on Emios Channel 3. |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 2 & 3 available. 2) Pin 34 & 35 in PortC available. 3) SCI Transceiver needed to corroborate the operation of reception bits. 4) Configure a Loop Back mode between TX and RX channels (Pins 34 & 35). |  |  | 1) EMSCI configuration structure properly initialized. 2) Pin 34 in PortC configured to operate in Output Compare Mode. 3) Pin 35 in PortC configured to operate on Input Capture Mode. | - Pin 34 in PortC configured to operate in Output Compare Mode. - Invoke the macro named “EMSCI_TX” using the below parameters: 2.1) Serial Unit number 0 2.2) 0x46. 3) Configure PortC pin 35 to operate in “Input Capture” mode. - After 100ms invoke macro function named “EMSCI_RX” using the below parameters: 4.1) SCI channel 1 4.2) Create a temporal variable named “u8RxData” to save any incoming data. 5) Run EMSCI driver. |  | 1)vfnEmsci_Report_Error Function should not be called. 2) After 100ms the variable named “u8RxData” shall have the value of 0x46 from RX unit 1. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC015 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 30 min |
| Objectives |  | Configure and Validate the RX functionality of RX Unit 2. Validate the correct reception of all bits on Emios Channel 5. |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 4 & 5 available. 2) Pin 36 & 37 in PortC available. 3) SCI Transceiver needed to corroborate the operation of reception bits. 4) Configure a Loop Back mode between TX and RX channels (Pins 36 & 37). |  |  | 1) EMSCI configuration structure properly initialized. 2) Pin 36 in PortC configured to operate in Output Compare Mode. 3) Pin 37 in PortC configured to operate on Input Capture Mode. | - Pin 36 in PortC configured to operate in Output Compare Mode. - Invoke the macro named “EMSCI_TX” using the below parameters: 2.1) Serial Unit number 0 2.2) 0x58. 3) Configure PortC pin 37 to operate in “Input Capture” mode. - After 200ms invoke macro function named “EMSCI_RX” using the below parameters: 4.1) SCI channel 2 4.2) Create a temporal variable named “u8RxData” to save any incoming data. 5) Run EMSCI driver. |  | 1)vfnEmsci_Report_Error Function should not be called. 2) After 200ms the variable named “u8RxData” shall have the value of 0x58 from RX unit 2. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | EMSCI_TC016 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 30 min |
| Objectives |  | Configure and Validate the RX functionality of RX Unit 3. Validate the correct reception of all bits on Emios Channel 7. |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Emios Channel 6 & 7 available. 2) Pin 38 & 39 in PortC available. 3) SCI Transceiver needed to corroborate the operation of reception bits. 4) Configure a Loop Back mode between TX and RX channels (Pins 38 & 39). |  |  | 1) EMSCI configuration structure properly initialized. 2) Pin 38 in PortC configured to operate in Output Compare Mode. 3) Pin 39 in PortC configured to operate on Input Capture Mode. | - Pin 38 in PortC configured to operate in Output Compare Mode. - Invoke the macro named “EMSCI_TX” using the below parameters: 2.1) Serial Unit number 3 2.2) 0x64. 3) Configure PortC pin 39 to operate in “Input Capture” mode. - After 400ms invoke macro function named “EMSCI_RX” using the below parameters: 4.1) SCI channel 3 4.2) Create a temporal variable named “u8RxData” to save any incoming data. 5) Run EMSCI driver. |  | 1)vfnEmsci_Report_Error Function should not be called. 2) After 200ms the variable named “u8RxData” shall have the value of 0x64 from RX unit 3. |  |
| Pass criteria |  |  |  |  |  |  |  |