RDB CPLD programming

Install Altera programmer:

Double click ‘QuartusProgrammerSetup’

Click Next

Select ‘I accept the agreement” and click Next.

Click next

Click next

Click Finish

Program CPLD:

Power down board and connect programmer. Power up board and launch ‘Quartus II 32-bit Programmer’.

Click on ‘Hardware Setup’

Double click USB-Blaster then close

Click ‘Add File’

Browse to file, select it and click open.

File shows in the window. Click check boxes ‘Program/configure’ and ‘verify’ then click ‘Start’. This programs the CPLD and verifies it.

Check the ‘Messages’ window that the verify was successful.

Power down board and unplug the programmer.